DecodeLabs

Robotics and Automation Internship Program

PROJECT 4

PLC-Based Conveyor Sorting System

I/O Mapping | Ladder / Structured Text Logic | State Machine | Safety Interlocks

(Optional Mastery Phase – Industrial Control & Safety)

Submitted By: Ayesha Nawaz

# 1. Introduction

This is the optional mastery-phase project of my Robotics and Automation internship at DecodeLabs: a PLC-Based Conveyor Sorting System. Unlike the earlier mobile-robot projects, this task moves into Industrial Control & Safety — designing the digital I/O map, sequence logic, and hardwired safety interlocks that a real Programmable Logic Controller (PLC) would run on a factory floor to sort boxes on a conveyor line.

Since real PLC hardware/software (e.g. Siemens TIA Portal, S7-PLCSIM) was not available, the exact same control logic was written twice for this report: once as IEC 61131-3 Structured Text (the direct textual equivalent of the Ladder Logic network that would be built in the PLC IDE), and once as a runnable Python simulation that models the PLC's deterministic scan cycle (Inputs → Logic → Outputs) so the behaviour of the design could actually be tested and verified.

# 2. Project Goal

Design the industrial control logic to sort boxes by size on an automated conveyor line.

# 3. Key Requirements

- Map digital Inputs (sensors) and Outputs (motors, pneumatic pushers).

- Write industrial Ladder Logic or a State Machine to control the sequence of operations.

- Implement strict safety interlocks (e.g., E-Stop logic halts all motors immediately).

Key Skills Practiced: PLC Programming basics, Ladder Logic, Sensor Integration, Industrial Safety Standards.

# 4. The Industrial Control Paradigm

A PLC repeatedly executes a deterministic cyclic scan: it reads all physical Inputs (sensors → 24VDC discrete signals), evaluates the Ladder/Structured Text Logic once per scan (typically 1ms–100ms), and then writes the Outputs (motors, solenoids) — converting digital decisions back into mechanical movement. This fixed cyclic-scan model is the key reason PLCs are preferred over general-purpose operating systems (like ROS) on the factory floor: it guarantees real-time, crash-free responsiveness.

# 5. Hardware Architecture and I/O Mapping

The physical inputs and outputs for the sorting station were mapped to discrete PLC tags as follows. A 500 ms software debounce filter is applied to the mechanical push-button inputs to prevent false state transitions caused by contact bounce.

| Tag | Symbol | Description |
| --- | --- | --- |
| %I0.0 | i_Start_PB | Start Push Button (NO) |
| %I0.1 | i_Prox_Sensor | Proximity Sensor – box present |
| %I0.2 | i_EStop_Mon | Safety Relay Monitor |
| %I0.3 | i_Stop_PB | Stop Push Button (NC) |
| %I0.4 | i_Tall_Sensor | High-Level Height Sensor |
| %Q1.0 | q_Conv_Motor | Belt Drive Contactor |
| %Q1.1 | q_Reject_Push | Diverter Solenoid Valve |
| %Q1.2 | q_Warn_Light | Amber Warning Beacon |

# 6. Sequence Logic: Finite State Machine

All control logic is routed through four mutually-exclusive states — Idle, Running, Sorting, and Fault — so the physical outputs can never receive conflicting commands (a 'race condition'). The system cannot jump directly from Idle to Sorting; it must always pass through the monitored Running state first.

Figure 1: Finite State Machine (FSM) topology showing all state transitions.

- Idle: motor off, pusher retracted, counter reset — waiting for the Start command.

- Running: motor energized (%Q1.0 = TRUE); actively monitoring the proximity sensor.

- Sorting: triggered by box detection; evaluates the height sensor and executes transit timing.

- Fault: triggered by E-Stop or a logic timeout; outputs are forced to a fail-safe state and the warning beacon is activated.

# 7. Edge Detection (Preventing Multi-Counting)

A box passing under the proximity sensor can remain 'seen' for many consecutive PLC scans (e.g. 40 scans for a box that blocks the beam for 400 ms at a 10 ms scan time). Evaluating the raw sensor level directly would count that single box 40 times. This is solved with an R_TRIG (rising/positive-edge) function block: Q_pulse = Input_current AND NOT(Input_previous), which produces exactly one single-scan pulse the instant the sensor first activates, regardless of how long the box remains under the beam.

Figure 2: Raw sensor signal (top) vs. the single-scan rising-edge pulse (bottom) used for counting.

# 8. Timers vs. Counters

Two different IEC function blocks are used depending on whether physical duration or discrete events are being tracked:

- TON (On-Delay Timer): measures elapsed physical duration. It requires a continuously-held input and resets automatically the moment that input goes FALSE. Used here to create the exact 1.5 s delay for a tall box to travel from the height sensor to the pneumatic pusher.

- CTU (Up-Counter): accumulates discrete events. It is driven by a transition-based rising-edge pulse (not a level) and only clears on an explicit RST command. Used here to count exactly 5 boxes to complete a batch.

Transit-timing calculation for the diverter pusher delay: the height sensor is mounted d = 0.75 m before the pusher, and the belt runs at v = 0.5 m/s, so the required on-delay is t = d / v = 0.75 / 0.5 = 1.5 seconds. The TON preset was therefore set to T#1500MS, so the pneumatic solenoid fires exactly when the box physically reaches the pusher — never too early, never missing it.

# 9. Safety Interlocks (E-Stop)

Per NFPA 79 / IEC 60204-1 style industrial safety standards, an Emergency Stop must never depend on software to cut power — if the PLC's CPU experiences a firmware lockup or memory fault, a purely software-based E-Stop would fail to de-energize the motors. The design therefore uses a dual-contact E-Stop button: one normally-closed contact is wired in series directly into the hardware safety relay, which physically cuts power to the motor contactor (Category 0 stop) completely independent of the PLC. A second contact is wired separately into a discrete PLC input (%I0.2) purely for diagnostics, HMI status display, and software latching — so the operator must perform an explicit manual reset before the line can restart, and the exact tripped E-Stop location can be shown on the HMI on a long line with multiple stations.

# 10. Structured Text (Ladder Logic Equivalent)

The Ladder Logic network for this station was written in its standard IEC 61131-3 Structured Text form below (this is the textual representation of the same rungs that would be built graphically in the PLC programming software, e.g. Siemens TIA Portal / Codesys).

(* ============================================================ PROJECT 4 : PLC-BASED CONVEYOR SORTING SYSTEM IEC 61131-3 STRUCTURED TEXT (equivalent of the Ladder Logic network that would be built in the PLC programming software) ============================================================ *) PROGRAM ConveyorSortingControl VAR (* ---- Digital Inputs (%I) ---- *) i_Start_PB : BOOL; (* %I0.0 Start Push Button (NO) *) i_Prox_Sensor : BOOL; (* %I0.1 Proximity Sensor *) i_EStop_Mon : BOOL; (* %I0.2 Safety Relay Monitor *) i_Stop_PB : BOOL; (* %I0.3 Stop Push Button (NC) *) i_Tall_Sensor : BOOL; (* %I0.4 High-Level Height Sensor *) (* ---- Digital Outputs (%Q) ---- *) q_Conv_Motor : BOOL; (* %Q1.0 Belt Drive Contactor *) q_Reject_Push : BOOL; (* %Q1.1 Diverter Solenoid Valve *) q_Warn_Light : BOOL; (* %Q1.2 Amber Warning Beacon *) (* ---- Internal state ---- *) state : INT := 1; (* 1=Idle 2=Running 3=Sorting 4=Fault *) b_EStop_Fault : BOOL := FALSE; b_Batch_Complete: BOOL := FALSE; (* ---- Function Blocks ---- *) BoxEdge : R_TRIG; (* rising-edge detector *) PushTimer : TON; (* on-delay timer *) BatchCounter : CTU; (* up-counter *) END_VAR (* 500 ms software debounce is applied upstream on i_Start_PB / i_Stop_PB inputs by the input-filter configuration of the physical channel. *) (* --------------------------------------------------------- SAFETY LAYER - evaluated first, every scan, highest priority. The E-Stop button itself is hard-wired through a dedicated safety relay directly into the motor contactor coil (Category 0 stop per IEC 60204-1) - this rung only LATCHES the software fault flag for diagnostics/HMI and forces safe PLC outputs. --------------------------------------------------------- *) IF NOT i_EStop_Mon THEN b_EStop_Fault := TRUE; END_IF; BoxEdge(CLK := i_Prox_Sensor); (* rising-edge pulse *) BatchCounter(CU := BoxEdge.Q, RESET := FALSE, PV := 5); IF BatchCounter.Q THEN b_Batch_Complete := TRUE; END_IF; (* --------------------------------------------------------- SEQUENCE-BASED STATE MACHINE --------------------------------------------------------- *) CASE state OF 1: (* IDLE *) q_Conv_Motor := FALSE; q_Reject_Push := FALSE; IF b_EStop_Fault THEN state := 4; ELSIF i_Start_PB AND NOT i_Stop_PB AND i_EStop_Mon THEN state := 2; (* T_1->2 *) END_IF; 2: (* RUNNING *) q_Conv_Motor := TRUE; IF b_EStop_Fault THEN state := 4; ELSIF i_Stop_PB THEN state := 1; ELSIF BoxEdge.Q AND NOT b_Batch_Complete THEN state := 3; (* T_2->3 *) END_IF; 3: (* SORTING *) q_Conv_Motor := TRUE; PushTimer(IN := i_Tall_Sensor, PT := T#1500MS); (* d/v = 0.75m/0.5m/s *) IF i_Tall_Sensor AND PushTimer.Q THEN q_Reject_Push := TRUE; (* fire pneumatic pusher *) ELSIF NOT i_Tall_Sensor THEN q_Reject_Push := FALSE; END_IF; IF b_EStop_Fault THEN state := 4; ELSIF NOT i_Tall_Sensor AND NOT i_Prox_Sensor THEN q_Reject_Push := FALSE; state := 2; (* box cleared -> back to Running *) END_IF; 4: (* FAULT - fail-safe, latched until manual reset *) q_Conv_Motor := FALSE; q_Reject_Push := FALSE; q_Warn_Light := TRUE; (* Operator must press a dedicated Reset button (wired to a separate %I) to clear b_EStop_Fault and return to state 1. *) END_CASE; q_Warn_Light := b_EStop_Fault; END_PROGRAM

# 11. Python Verification Model

Since real PLC hardware / PLCSIM was unavailable, the exact same I/O map, FSM, R_TRIG, TON, and CTU logic above was re-implemented as a runnable Python model of the PLC's scan cycle, so the design could be verified against a full test sequence (startup, short box, tall box requiring the timed pusher, batch counting, an E-Stop event, and a safe manual restart).

""" ================================================================================ PROJECT 4: PLC-BASED CONVEYOR SORTING SYSTEM (Optional Mastery Phase) Industrial Training Kit - DecodeLabs Robotics & Automation Internship ================================================================================ GOAL: Design the industrial control logic to sort boxes by size on an automated conveyor line, using a PLC-style scan-cycle architecture. This script simulates a real PLC's cyclic scan (Inputs -> Logic -> Outputs) in Python. The control logic itself is written exactly as it would be structured on a real PLC (Finite State Machine + rising-edge detection + TON timer + CTU counter + hardwired E-Stop interlock), so this file is a faithful, runnable model of the Structured Text / Ladder Logic listed in the report. KEY REQUIREMENTS IMPLEMENTED: 1. Digital I/O map (sensors -> inputs, motors/pushers -> outputs) 2. Sequence-based Finite State Machine (Idle -> Running -> Sorting -> Idle) 3. Strict E-Stop safety interlock (hardware-priority, halts all motors) KEY SKILLS DEMONSTRATED: PLC programming basics, Ladder/Structured Text logic, Sensor integration, Industrial safety standards (NFPA 79 / IEC 60204-1 style). Author : Ayesha Nawaz (BS Robotics & Intelligent Systems, NUML Islamabad) Internship : Robotics and Automation, DecodeLabs ================================================================================ """ import matplotlib.pyplot as plt import matplotlib.patches as mpatches from matplotlib.patches import FancyArrowPatch, FancyBboxPatch, Rectangle import os OUT = os.path.join(os.path.dirname(__file__), "images") os.makedirs(OUT, exist_ok=True) # ------------------------------------------------------------------------ # 1. DIGITAL I/O MAP (%I = discrete input, %Q = discrete output) # ------------------------------------------------------------------------ IO_MAP = { "inputs": [ ("%I0.0", "i_Start_PB", "Start Push Button (NO)"), ("%I0.1", "i_Prox_Sensor", "Proximity Sensor - box present"), ("%I0.2", "i_EStop_Mon", "Safety Relay Monitor (E-Stop)"), ("%I0.3", "i_Stop_PB", "Stop Push Button (NC)"), ("%I0.4", "i_Tall_Sensor", "High-Level Height Sensor"), ], "outputs": [ ("%Q1.0", "q_Conv_Motor", "Belt Drive Contactor"), ("%Q1.1", "q_Reject_Push", "Diverter Solenoid Valve (pneumatic pusher)"), ("%Q1.2", "q_Warn_Light", "Amber Warning Beacon"), ], } # Physical transit-timing calculation (from sensor to pusher): # distance d = 0.75 m, belt speed v = 0.5 m/s -> t = d / v TRANSIT_DISTANCE_M = 0.75 BELT_SPEED_MPS = 0.5 TRANSIT_DELAY_S = TRANSIT_DISTANCE_M / BELT_SPEED_MPS # 1.5 s DEBOUNCE_S = 0.5 # 500 ms software debounce on mechanical buttons BATCH_SIZE = 5 # boxes per batch (CTU preset) # ------------------------------------------------------------------------ # 2. TIMER / COUNTER / EDGE-DETECTION BLOCKS (IEC 61131-3 style) # ------------------------------------------------------------------------ class R_TRIG: """Rising-edge (positive edge) detector. Fires TRUE for exactly one scan when the input transitions FALSE -> TRUE. Prevents a single box sitting under a sensor for many scans from being counted multiple times.""" def __init__(self): self.prev = False def scan(self, current: bool) -> bool: pulse = current and not self.prev self.prev = current return pulse class TON: """On-Delay Timer. Measures elapsed physical duration while its input (IN) is held TRUE. Q becomes TRUE once elapsed time (ET) >= preset time (PT). Resets automatically the instant IN goes FALSE.""" def __init__(self, preset_s: float): self.PT = preset_s self.ET = 0.0 self.Q = False def scan(self, IN: bool, dt: float): if IN: self.ET = min(self.ET + dt, self.PT) self.Q = self.ET >= self.PT else: self.ET = 0.0 self.Q = False return self.Q class CTU: """Up-Counter. Accumulates discrete rising-edge events (CV) until it reaches the preset value (PV), then Q latches TRUE. Only an explicit RST command clears it - it does NOT reset automatically like a TON.""" def __init__(self, preset: int): self.PV = preset self.CV = 0 self.Q = False def scan(self, CU_pulse: bool, RST: bool): if RST: self.CV = 0 self.Q = False elif CU_pulse: self.CV += 1 self.Q = self.CV >= self.PV return self.Q # ------------------------------------------------------------------------ # 3. FINITE STATE MACHINE (mutually-exclusive states -> no race conditions) # ------------------------------------------------------------------------ IDLE, RUNNING, SORTING, FAULT = "IDLE", "RUNNING", "SORTING", "FAULT" class ConveyorSortingPLC: """ Faithful simulation of the PLC's cyclic scan for the conveyor sorting station. Every call to scan_cycle() represents ONE PLC scan (inputs are read, logic (this method) is evaluated, outputs are written) - exactly matching the deterministic Input->Process->Output cycle shown in the training material. """ def __init__(self): self.state = IDLE self.box_edge = R_TRIG() self.push_timer = TON(preset_s=TRANSIT_DELAY_S) self.batch_counter = CTU(preset=BATCH_SIZE) self.b_estop_fault = False self.b_batch_complete = False self.log = [] # Outputs (physical world) self.q_conv_motor = False self.q_reject_push = False self.q_warn_light = False def scan_cycle(self, i_start_pb, i_prox_sensor, i_estop_mon, i_stop_pb, i_tall_sensor, dt=0.1): """One PLC scan. dt = simulated scan/time-step in seconds.""" # ---- SAFETY LAYER (hardware-priority, evaluated every scan) ---- # NOTE: in the real system the E-Stop button physically breaks the # safety relay -> motor contactor BEFORE the PLC even sees it. # i_estop_mon simply reports relay health back to the PLC for # diagnostics/latching, matching "Mandatory: Hardware Cutoff". if not i_estop_mon: self.b_estop_fault = True # ---- Rising-edge pulse for box counting (prevents multi-count) -- box_pulse = self.box_edge.scan(i_prox_sensor) # ---- Batch counter runs off the edge pulse every scan, independent # of which FSM state is active this scan (a real CTU block is # wired directly to the sensor pulse, not gated by the FSM) -- if self.batch_counter.scan(box_pulse, RST=False): if not self.b_batch_complete: self.log.append(f" -> Batch counter reached preset " f"({self.batch_counter.CV}/{self.batch_counter.PV})") self.b_batch_complete = True # ---- FSM TRANSITIONS ---- if self.b_estop_fault: self.state = FAULT elif self.state == IDLE: # T_1->2 : start pressed, stop not pressed, e-stop healthy, no fault if i_start_pb and not i_stop_pb and i_estop_mon and not self.b_estop_fault: self.state = RUNNING self.q_conv_motor = True self.log.append("IDLE -> RUNNING (start command accepted)") elif self.state == RUNNING: self.q_conv_motor = True if i_stop_pb: self.state = IDLE self.q_conv_motor = False self.log.append("RUNNING -> IDLE (operator stop)") # T_2->3 : proximity sensor fires, no fault, batch not complete elif box_pulse and not self.b_estop_fault and not self.b_batch_complete: self.state = SORTING self.log.append(f"RUNNING -> SORTING (box detected, edge pulse)") elif self.state == SORTING: self.q_conv_motor = True # Only tall boxes are diverted; short boxes pass straight through. timer_done = self.push_timer.scan(i_tall_sensor, dt) if i_tall_sensor: if timer_done: self.q_reject_push = True self.log.append(" -> Tall box confirmed: TON elapsed " f"({self.push_timer.ET:.2f}s/{self.push_timer.PT:.2f}s) " "-> Diverter pusher FIRED") else: self.q_reject_push = False # once the box has cleared the diverter, return to Running if not i_tall_sensor and not i_prox_sensor: self.q_reject_push = False self.state = RUNNING self.log.append("SORTING -> RUNNING (box cleared)") # ---- FAULT STATE : outputs forced to fail-safe ---- if self.state == FAULT: self.q_conv_motor = False self.q_reject_push = False self.q_warn_light = True else: self.q_warn_light = self.b_estop_fault return self.state def reset_after_estop(self): """Manual operator reset required before restart (latching safety).""" self.b_estop_fault = False self.state = IDLE self.log.append("Manual RESET performed -> FAULT cleared, system in IDLE") # ------------------------------------------------------------------------ # 4. DEMONSTRATION SCAN SEQUENCE # ------------------------------------------------------------------------ def run_demo(): plc = ConveyorSortingPLC() print("=" * 72) print(" PLC CONVEYOR SORTING SYSTEM - SCAN CYCLE SIMULATION") print("=" * 72) trace = [] # for the state-timeline plot: (scan_no, state) def step(label, **inputs): state = plc.scan_cycle(**inputs) trace.append(state) print(f"[{label:>16}] state={state:<8} " f"Q_motor={int(plc.q_conv_motor)} " f"Q_push={int(plc.q_reject_push)} " f"Q_warn={int(plc.q_warn_light)}") # -- 1) System starts in IDLE, operator presses Start -- step("startup", i_start_pb=True, i_prox_sensor=False, i_estop_mon=True, i_stop_pb=False, i_tall_sensor=False) # -- 2) Conveyor running, a SHORT box passes (no diversion expected) -- step("short-box-in", i_start_pb=False, i_prox_sensor=True, i_estop_mon=True, i_stop_pb=False, i_tall_sensor=False) step("short-box-out", i_start_pb=False, i_prox_sensor=False, i_estop_mon=True, i_stop_pb=False, i_tall_sensor=False) # -- 3) A TALL box arrives: height sensor active, TON must elapse 1.5s # before the diverter pusher fires -- step("tall-box-in", i_start_pb=False, i_prox_sensor=True, i_estop_mon=True, i_stop_pb=False, i_tall_sensor=True) for k in range(16): # 16 scans x 0.1s = 1.6s > 1.5s preset step(f"tall-wait-{k+1}", i_start_pb=False, i_prox_sensor=True, i_estop_mon=True, i_stop_pb=False, i_tall_sensor=True) step("tall-box-out", i_start_pb=False, i_prox_sensor=False, i_estop_mon=True, i_stop_pb=False, i_tall_sensor=False) # -- 4) Two more short boxes to build up the batch counter -- for n in range(2): step(f"box{n+3}-in", i_start_pb=False, i_prox_sensor=True, i_estop_mon=True, i_stop_pb=False, i_tall_sensor=False) step(f"box{n+3}-out", i_start_pb=False, i_prox_sensor=False, i_estop_mon=True, i_stop_pb=False, i_tall_sensor=False) # -- 5) EMERGENCY STOP is pressed mid-operation -- step("E-STOP PRESSED", i_start_pb=False, i_prox_sensor=False, i_estop_mon=False, i_stop_pb=False, i_tall_sensor=False) step("still-faulted", i_start_pb=True, i_prox_sensor=False, i_estop_mon=False, i_stop_pb=False, i_tall_sensor=False) # -- 6) Operator manually resets after clearing the hazard -- plc.reset_after_estop() trace.append(plc.state) step("restart-ok", i_start_pb=True, i_prox_sensor=False, i_estop_mon=True, i_stop_pb=False, i_tall_sensor=False) print("\n--- Event Log ---") for entry in plc.log: print(" -", entry) print(f"\nBatch counter final value: {plc.batch_counter.CV}/{plc.batch_counter.PV}") print(f"Transit delay used by TON : {TRANSIT_DELAY_S:.2f} s " f"(= {TRANSIT_DISTANCE_M} m / {BELT_SPEED_MPS} m/s)") print("=" * 72) return plc, trace # ------------------------------------------------------------------------ # 5. DIAGRAMS # ------------------------------------------------------------------------ def draw_io_table(): fig, ax = plt.subplots(figsize=(9, 5)) ax.axis("off") ax.set_title("Hardware Architecture & I/O Mapping", fontsize=15, fontweight="bold", pad=20) col_headers = ["Tag", "Symbol", "Description"] rows = [] for tag, sym, desc in IO_MAP["inputs"]: rows.append((tag, sym, desc, "#0f6fa8")) sep = len(rows) for tag, sym, desc in IO_MAP["outputs"]: rows.append((tag, sym, desc, "#2e8b3d")) n = len(rows) row_h = 1.0 / (n + 1) col_x = [0.02, 0.18, 0.42] ax.text(col_x[0], 1 - row_h * 0.5, "Tag", fontweight="bold", fontsize=11) ax.text(col_x[1], 1 - row_h * 0.5, "Symbol", fontweight="bold", fontsize=11) ax.text(col_x[2], 1 - row_h * 0.5, "Description", fontweight="bold", fontsize=11) ax.plot([0, 1], [1 - row_h, 1 - row_h], color="black", linewidth=1) for idx, (tag, sym, desc, color) in enumerate(rows): y = 1 - row_h * (idx + 1.5) if idx == sep: ax.plot([0, 1], [y + row_h * 0.5, y + row_h * 0.5], color="grey", linewidth=0.8, linestyle="--") ax.text(col_x[0], y, tag, fontsize=10, color=color, fontfamily="monospace", fontweight="bold") ax.text(col_x[1], y, sym, fontsize=10, fontfamily="monospace") ax.text(col_x[2], y, desc, fontsize=10) ax.text(0.02, -0.02, "Blue = Digital Inputs ¦ Green = Digital Outputs", fontsize=9, color="#555555", transform=ax.transAxes) plt.tight_layout() plt.savefig(f"{OUT}/1_io_mapping.png", dpi=150) plt.close() def draw_fsm_diagram(): fig, ax = plt.subplots(figsize=(7, 7)) ax.set_xlim(0, 10) ax.set_ylim(0, 10) ax.axis("off") ax.set_title("Finite State Machine (FSM) Topology", fontsize=14, fontweight="bold") boxes = { "IDLE": (5, 8.3, "#777777", "Motor off, pusher retracted,\ncounter reset"), "RUNNING": (8, 5.5, "#2e8b3d", "Motor energized (%Q1.0=1)\nmonitoring sensors"), "SORTING": (5, 2.7, "#b8860b", "Box detected, evaluate height\n& execute transit timing"), "FAULT": (2, 5.5, "#c0392b", "E-Stop / timeout\noutputs forced fail-safe"), } for name, (x, y, color, desc) in boxes.items(): box = FancyBboxPatch((x - 1.3, y - 0.7), 2.6, 1.4, boxstyle="round,pad=0.05,rounding_size=0.15", linewidth=2, edgecolor=color, facecolor="#ffffff") ax.add_patch(box) ax.text(x, y + 0.25, name, ha="center", fontsize=12, fontweight="bold", color=color) ax.text(x, y - 0.28, desc, ha="center", fontsize=7.5, color="#333333") def arrow(p1, p2, label, color="black", rad=0.15): a = FancyArrowPatch(p1, p2, connectionstyle=f"arc3,rad={rad}", arrowstyle="-¦>", mutation_scale=16, color=color, linewidth=1.6) ax.add_patch(a) mx, my = (p1[0] + p2[0]) / 2, (p1[1] + p2[1]) / 2 ax.text(mx, my + 0.35 * (1 if rad >= 0 else -1), label, fontsize=7.5, ha="center", color=color) arrow((6.1, 8.0), (8.5, 6.3), "Start_PB", "#2e8b3d") arrow((7.3, 4.9), (5.8, 3.2), "Box sensor\n(edge pulse)", "#b8860b") arrow((4.2, 3.2), (3.2, 5.0), "Batch complete", "#555555", rad=0.2) arrow((3.7, 6.2), (4.2, 8.0), "Reset (manual)", "#555555", rad=0.2) for src in [(4.4, 8.0), (7.0, 5.8), (4.4, 2.9)]: arrow(src, (2.7, 5.9), "E-Stop", "#c0392b", rad=-0.1) plt.tight_layout() plt.savefig(f"{OUT}/2_fsm_diagram.png", dpi=150) plt.close() def draw_state_timeline(trace): order = {IDLE: 0, RUNNING: 1, SORTING: 2, FAULT: 3} y = [order[s] for s in trace] x = list(range(len(trace))) fig, ax = plt.subplots(figsize=(11, 3.2)) colors = {"IDLE": "#777777", "RUNNING": "#2e8b3d", "SORTING": "#b8860b", "FAULT": "#c0392b"} ax.step(x, y, where="post", color="#333333", linewidth=1.5) for i, s in enumerate(trace): ax.scatter([i], [order[s]], color=colors[s], s=35, zorder=5) ax.set_yticks([0, 1, 2, 3]) ax.set_yticklabels(["IDLE", "RUNNING", "SORTING", "FAULT"]) ax.set_xlabel("PLC Scan Number") ax.set_title("FSM State Timeline During Simulated Run", fontsize=13, fontweight="bold") ax.grid(axis="y", linestyle="--", alpha=0.4) plt.tight_layout() plt.savefig(f"{OUT}/3_state_timeline.png", dpi=150) plt.close() def draw_edge_detection(): import numpy as np fig, axes = plt.subplots(2, 1, figsize=(9, 5), sharex=True) t = np.linspace(0, 1.0, 400) sensor = ((t > 0.2) & (t < 0.6)).astype(float) axes[0].plot(t, sensor, color="#c0392b", linewidth=1.8) axes[0].fill_between(t, sensor, alpha=0.15, color="#c0392b") axes[0].set_title("Sensor Obstructed (static level = many TRUE scans)", fontsize=10, fontweight="bold") axes[0].set_ylim(-0.2, 1.3) axes[0].set_yticks([0, 1]) pulse = np.zeros_like(t) idx = np.argmax(sensor) pulse[idx:idx + 4] = 1 axes[1].plot(t, pulse, color="#2e8b3d", linewidth=1.8) axes[1].fill_between(t, pulse, alpha=0.25, color="#2e8b3d") axes[1].set_title("R_TRIG Output: exactly one 1-scan pulse per box", fontsize=10, fontweight="bold") axes[1].set_ylim(-0.2, 1.3) axes[1].set_yticks([0, 1]) axes[1].set_xlabel("Time (s)") plt.tight_layout() plt.savefig(f"{OUT}/4_edge_detection.png", dpi=150) plt.close() if __name__ == "__main__": draw_io_table() draw_fsm_diagram() draw_edge_detection() plc, trace = run_demo() draw_state_timeline(trace) print("\nAll diagrams saved to:", OUT)

# 12. Simulation Results

Running the verification model against the test sequence produced the following state timeline and scan-by-scan log:

Figure 3: FSM state timeline across the full simulated scan sequence.

[ startup] state=RUNNING Q_motor=1 Q_push=0 Q_warn=0 [ short-box-in] state=SORTING Q_motor=1 Q_push=0 Q_warn=0 [ short-box-out] state=RUNNING Q_motor=1 Q_push=0 Q_warn=0 [ tall-box-in] state=SORTING Q_motor=1 Q_push=0 Q_warn=0 ... (TON accumulating: ET rising toward 1.50 s preset) ... [ tall-wait-15] state=SORTING Q_motor=1 Q_push=1 Q_warn=0 <- pusher fires at 1.50s [ tall-wait-16] state=SORTING Q_motor=1 Q_push=1 Q_warn=0 [ tall-box-out] state=RUNNING Q_motor=1 Q_push=0 Q_warn=0 [ box3-in] state=SORTING Q_motor=1 Q_push=0 Q_warn=0 [ box3-out] state=RUNNING Q_motor=1 Q_push=0 Q_warn=0 [ box4-in] state=SORTING Q_motor=1 Q_push=0 Q_warn=0 [ box4-out] state=RUNNING Q_motor=1 Q_push=0 Q_warn=0 [ E-STOP PRESSED] state=FAULT Q_motor=0 Q_push=0 Q_warn=1 <- all motors halted instantly [ still-faulted] state=FAULT Q_motor=0 Q_push=0 Q_warn=1 <- Start ignored while faulted [ restart-ok] state=RUNNING Q_motor=1 Q_push=0 Q_warn=0 <- after manual reset Batch counter final value : 4/5 Transit delay used by TON : 1.50 s (= 0.75 m / 0.5 m/s)

The results confirm all three key requirements: the I/O map correctly drives the physical outputs, the sequence-based state machine (with rising-edge box detection, a 1.5 s TON transit delay, and a 5-item CTU batch counter) controls the full sort operation, and the E-Stop interlock immediately forces Q_motor and Q_push to 0 the instant it is tripped — with the system correctly refusing to restart (Start_PB is ignored) until a deliberate manual reset is performed.

# 13. Conclusion

This project gave me hands-on exposure to industrial control design, which is quite different from robotics-side programming: instead of continuous motion planning, PLC logic is about deterministic scan cycles, discrete I/O, and above all, safety-first design. Building the FSM taught me why states must be mutually exclusive, edge detection taught me why level-based sensing silently breaks counters, and the E-Stop design taught me why safety-critical stops must never be purely software-dependent. Completing this optional milestone adds Industrial Automation to my verified skill set alongside the SLAM/pathfinding work from Project 3, and gives me a much fuller picture of how a real automated line — from sensor to PLC to actuator — is actually built.