Robotics & Automation Internship

Project 2 - Complete Report

Automated Quality Inspection using Computer Vision

Submitted By : Ayesha Nawaz

# 1. Introduction

For this project, I worked as a Robotics and Automation Engineer intern at DecodeLabs on Project 2: Automated Quality Inspection using Computer Vision. In factories, parts move on a conveyor belt and someone has to check every single part for defects. A human inspector gets tired after checking hundreds of parts and can easily miss a small defect. My job in this project was to build a Python program using OpenCV that looks at an image of a gear and automatically decides if it is a good part (PASS) or a defective part (FAIL), without any human help.

# 2. Objective

- Process gear images using OpenCV image processing functions.

- Apply pre-processing: Grayscale conversion, Gaussian Blur, and Thresholding.

- Find the outer boundary (contour) of each gear and compare it with its convex hull.

- Measure the gap between the contour and the hull (convexity defects) and use it to decide PASS or FAIL.

- Draw a bounding box on the exact location of any detected defect.

# 3. Dataset Used

The project brief asked for a 20-image validation dataset with 10 good/control parts and 10 defective parts. Since a real factory dataset was not given to me, I wrote a small Python script (generate_dataset.py, included in full below) that programmatically draws 20 gear images - 10 with all teeth intact, and 10 with one broken/missing tooth to simulate a real defect. This let me test my complete pipeline end-to-end exactly the way it would run on real camera images. If a real dataset is provided later, the exact same detection code works directly on it - only the image file paths need to change.

# 4. Methodology (IPO Architecture)

I followed the Input-Process-Output architecture explained in the project guide. Below I explain each stage and show the exact code used for it.

## 4.1 Input & Pre-processing

First, the image is read and converted from color (BGR) to Grayscale using cv2.cvtColor(). This reduces the image to a single intensity channel, which is enough for shape detection. Then I applied cv2.GaussianBlur() to smooth out small noise (like dust or lighting variation) so it doesn't get mistaken for a real defect. Finally, cv2.threshold() converts the smoothed grayscale image into a clean black-and-white (binary) image, where the gear appears as a solid white shape on a black background.

Code used for this stage:

def preprocess_image(image):
    gray = cv2.cvtColor(image, cv2.COLOR_BGR2GRAY)
    blurred = cv2.GaussianBlur(gray, (5, 5), 0)
    _, thresh = cv2.threshold(blurred, 60, 255, cv2.THRESH_BINARY)
    return thresh

## 4.2 Finding the Boundary

Once I have the clean binary image, I use cv2.findContours() with the RETR_EXTERNAL flag. This flag is important because it tells OpenCV to trace only the outermost boundary of the gear and to ignore internal features like the center shaft hole. The result is a list of (x, y) points forming the outline of the gear. I then pick the largest contour found, since tiny noise specks could also appear as very small contours.

Code used for this stage:

def get_largest_contour(thresh_img):
    contours, _ = cv2.findContours(thresh_img, cv2.RETR_EXTERNAL,
                                    cv2.CHAIN_APPROX_SIMPLE)
    if not contours:
        return None
    largest = max(contours, key=cv2.contourArea)
    return largest

## 4.3 Convex Hull and Convexity Defects

To detect a broken tooth, I compare the actual gear outline with its Convex Hull - the smallest possible shape that completely wraps around the gear, like stretching a rubber band around it. Anywhere the real outline dips in more than expected compared to this rubber band is called a convexity defect. A normal gap between two teeth causes a small, consistent defect depth. A broken or missing tooth causes a much deeper, larger defect because the material is simply not there anymore.

One important detail I learned is that OpenCV returns the defect distance scaled by a fixed value of 256, so the raw number must be divided by 256.0 to get the actual pixel distance. I made sure to include this division in my code, otherwise the numbers would come out 256 times too large.

Code used for this stage:

def find_worst_defect(contour):
    hull = cv2.convexHull(contour, returnPoints=False)
    if hull is None or len(hull) < 3:
        return None
    try:
        defects = cv2.convexityDefects(contour, hull)
    except cv2.error:
        return None
    if defects is None:
        return None
 
    worst_defect = None
    max_distance = 0
    for i in range(defects.shape[0]):
        start_idx, end_idx, farthest_idx, d_raw = defects[i, 0]
        actual_distance = d_raw / 256.0   # fixed-point correction
        if actual_distance > max_distance:
            max_distance = actual_distance
            farthest_point = tuple(contour[farthest_idx][0])
            worst_defect = {"distance": actual_distance, "point": farthest_point}
    return worst_defect

## 4.4 The Tolerance Gate (Decision Logic)

After getting the real defect distance in pixels, I compare it against a threshold value (THRESHOLD_MAX = 35 pixels in my code). If the deepest defect found on a gear is below this threshold, it means the gap is just a normal tooth gap, so the part is marked PASS. If the defect distance goes above the threshold, it means there is a real structural problem (a broken tooth), so the part is marked FAIL, and a red bounding box is drawn around the exact location of the fault using cv2.rectangle().

Code used for this stage:

if defect is None or defect["distance"] <= THRESHOLD_MAX:
    verdict = "PASS"
else:
    verdict = "FAIL"
    px, py = defect["point"]
    top_left = (px - 25, py - 25)
    bottom_right = (px + 25, py + 25)
    cv2.rectangle(result_img, top_left, bottom_right, (0, 0, 255), 2)

# 5. Results

I ran the pipeline on all 20 images in my dataset. Below is the full result table for all 20 images:

| Image | Expected | System Output | Defect (px) | Result |
| --- | --- | --- | --- | --- |
| good_1.jpg | PASS | PASS | 26.9 | OK |
| good_2.jpg | PASS | PASS | 26.9 | OK |
| good_3.jpg | PASS | PASS | 26.9 | OK |
| good_4.jpg | PASS | PASS | 26.9 | OK |
| good_5.jpg | PASS | PASS | 26.9 | OK |
| good_6.jpg | PASS | PASS | 26.9 | OK |
| good_7.jpg | PASS | PASS | 26.9 | OK |
| good_8.jpg | PASS | PASS | 26.9 | OK |
| good_9.jpg | PASS | PASS | 26.9 | OK |
| good_10.jpg | PASS | PASS | 26.9 | OK |
| bad_1.jpg | FAIL | FAIL | 48.3 | OK |
| bad_2.jpg | FAIL | FAIL | 49.5 | OK |
| bad_3.jpg | FAIL | FAIL | 48.9 | OK |
| bad_4.jpg | FAIL | FAIL | 49.0 | OK |
| bad_5.jpg | FAIL | FAIL | 48.3 | OK |
| bad_6.jpg | FAIL | FAIL | 49.0 | OK |
| bad_7.jpg | FAIL | FAIL | 48.1 | OK |
| bad_8.jpg | FAIL | FAIL | 49.5 | OK |
| bad_9.jpg | FAIL | FAIL | 48.3 | OK |
| bad_10.jpg | FAIL | FAIL | 48.1 | OK |

Out of 20 images, the system correctly classified all 20 (100% accuracy) once the threshold value was properly tuned. Below are sample output images showing two PASS results and two FAIL results, with the defect bounding box drawn on the FAIL ones:

# 6. Challenges Faced

- Choosing the right threshold value: at first my threshold was too low, so even the normal keyway notch on a good gear was being flagged as a defect. I fixed this by testing different threshold values and picking one that separates normal gaps from real defects.

- Understanding the 256 scaling factor returned by cv2.convexityDefects() - I had to divide the raw distance by 256.0 to get the correct pixel value.

- Making sure RETR_EXTERNAL was used so the center hole of the gear was not mistakenly treated as a defect.

# 7. Conclusion

Through this project, I learned how a computer vision system can replace manual visual inspection in a factory setting. By breaking the problem into small stages - cleaning the image, finding its outline, comparing it to an ideal convex shape, and applying a tolerance check - I was able to build a system that reliably tells apart good parts from defective ones. This project helped me understand OpenCV functions like cvtColor, GaussianBlur, threshold, findContours, convexHull, and convexityDefects, and how they work together to give a machine the ability to 'see' and make a pass/fail decision on its own.

# Appendix A: Full Source Code - defect_detection.py

This is the complete, working inspection pipeline. This is the main file that should be run to get the PASS/FAIL results.

"""
====================================================================
 Project 2 : Automated Quality Inspection (Computer Vision)
 Robotics & Automation - DecodeLabs Internship
 Name   : Ayesha
 Roll No: I-BS-RISA-2260004
====================================================================
 
Goal:
    Build an automated optical inspection system that looks at gear
    images coming from a conveyor belt and decides PASS or FAIL by
    detecting broken teeth / structural defects using OpenCV.
 
Pipeline followed (IPO Architecture from project brief):
    INPUT   -> capture image, convert to grayscale
    PROCESS -> Gaussian Blur -> Threshold -> Find Contours ->
               Convex Hull -> Convexity Defects
    OUTPUT  -> compare defect distance with a tolerance value,
               draw bounding box on the fault, print PASS/FAIL
====================================================================
"""
 
import cv2
import numpy as np
import os
 
# ---------------------------------------------------------------
# CONFIGURATION
# ---------------------------------------------------------------
DATASET_DIR = "dataset"
OUTPUT_DIR = "output"
os.makedirs(OUTPUT_DIR, exist_ok=True)
 
# This is the tolerance gate mentioned in Phase 3 of the brief.
# Any convexity defect deeper than this value (in pixels) is
# treated as a real structural fault (broken tooth / crack),
# not just the normal gap between two gear teeth.
THRESHOLD_MAX = 35.0
 
 
def preprocess_image(image):
    """
    PHASE 1: Isolating the signal from the noise.
    Converts the raw BGR image into a clean binary silhouette.
    """
    gray = cv2.cvtColor(image, cv2.COLOR_BGR2GRAY)
 
    # Gaussian blur smooths out sensor noise / dust before we
    # try to find sharp edges.
    blurred = cv2.GaussianBlur(gray, (5, 5), 0)
 
    # Binarize: pixels above 60 become white (the gear), rest black.
    # (threshold value tuned for our sample images - darker background)
    _, thresh = cv2.threshold(blurred, 60, 255, cv2.THRESH_BINARY)
 
    return thresh
 
 
def get_largest_contour(thresh_img):
    """
    PHASE 2 - Step 1: Tracing the boundary.
    RETR_EXTERNAL makes sure we only get the outer shape of the
    gear, ignoring the center shaft hole.
    """
    contours, _ = cv2.findContours(thresh_img, cv2.RETR_EXTERNAL,
                                    cv2.CHAIN_APPROX_SIMPLE)
    if not contours:
        return None
 
    # pick the biggest contour = the gear itself (ignores tiny noise blobs)
    largest = max(contours, key=cv2.contourArea)
    return largest
 
 
def find_worst_defect(contour):
    """
    PHASE 2 - Step 2 & 3: The Rubber Band Concept + Measuring the Gaps.
 
    Builds a convex hull ("rubber band") around the gear contour,
    then finds convexity defects (gaps between contour and hull).
    Returns the single worst (deepest) defect found.
    """
    hull = cv2.convexHull(contour, returnPoints=False)
 
    # Safety check: convexHull needs at least 3 points and the
    # points must be in a valid order, otherwise convexityDefects fails.
    if hull is None or len(hull) < 3:
        return None
 
    try:
        defects = cv2.convexityDefects(contour, hull)
    except cv2.error:
        return None
 
    if defects is None:
        return None
 
    worst_defect = None
    max_distance = 0
 
    for i in range(defects.shape[0]):
        start_idx, end_idx, farthest_idx, d_raw = defects[i, 0]
 
        # IMPORTANT (Pro-Tip from brief): OpenCV returns distance
        # scaled by a fixed-point integer of 256, so we must divide.
        actual_distance = d_raw / 256.0
 
        if actual_distance > max_distance:
            max_distance = actual_distance
            farthest_point = tuple(contour[farthest_idx][0])
            worst_defect = {
                "distance": actual_distance,
                "point": farthest_point,
            }
 
    return worst_defect
 
 
def inspect_image(image_path):
    """
    PHASE 3: The Tolerance Gate.
    Runs the full pipeline on one image and returns the verdict
    plus an annotated copy of the image.
    """
    image = cv2.imread(image_path)
    if image is None:
        return "ERROR", None, 0.0
 
    thresh = preprocess_image(image)
    contour = get_largest_contour(thresh)
 
    result_img = image.copy()
 
    if contour is None:
        cv2.putText(result_img, "NO PART DETECTED", (10, 30),
                    cv2.FONT_HERSHEY_SIMPLEX, 0.7, (0, 0, 255), 2)
        return "FAIL", result_img, 0.0
 
    defect = find_worst_defect(contour)
 
    if defect is None or defect["distance"] <= THRESHOLD_MAX:
        verdict = "PASS"
        color = (0, 255, 0)
        distance = 0.0 if defect is None else defect["distance"]
    else:
        verdict = "FAIL"
        color = (0, 0, 255)
        distance = defect["distance"]
 
        # Draw a bounding box around the defect location so an
        # operator can instantly see WHERE the fault is.
        px, py = defect["point"]
        top_left = (px - 25, py - 25)
        bottom_right = (px + 25, py + 25)
        cv2.rectangle(result_img, top_left, bottom_right, (0, 0, 255), 2)
 
    # Overlay the verdict text on the image
    cv2.putText(result_img, f"{verdict} (defect={distance:.1f}px)",
                (10, 30), cv2.FONT_HERSHEY_SIMPLEX, 0.6, color, 2)
 
    return verdict, result_img, distance
 
 
def run_full_dataset():
    """
    Runs inspection on every image inside dataset/ and prints a
    summary table, then saves annotated images into output/.
    """
    files = sorted(os.listdir(DATASET_DIR))
    correct = 0
    total = 0
 
    print(f"{'File':<15}{'Expected':<12}{'Verdict':<10}{'Defect(px)':<12}{'Result'}")
    print("-" * 60)
 
    for f in files:
        if not f.lower().endswith((".jpg", ".png", ".jpeg")):
            continue
 
        expected = "PASS" if f.startswith("good") else "FAIL"
        path = os.path.join(DATASET_DIR, f)
        verdict, annotated, distance = inspect_image(path)
 
        is_correct = (verdict == expected)
        correct += int(is_correct)
        total += 1
 
        print(f"{f:<15}{expected:<12}{verdict:<10}{distance:<12.1f}"
              f"{'OK' if is_correct else 'WRONG'}")
 
        if annotated is not None:
            cv2.imwrite(os.path.join(OUTPUT_DIR, f), annotated)
 
    accuracy = (correct / total) * 100 if total else 0
    print("-" * 60)
    print(f"Accuracy: {correct}/{total} = {accuracy:.1f}%")
 
 
if __name__ == "__main__":
    run_full_dataset()

# Appendix B: Full Source Code - generate_dataset.py

This script was used only to create the 20 sample test images (10 good, 10 defective), since a real factory dataset was not available. It does not need to be run again if real images are supplied.

"""
generate_dataset.py
--------------------
Creates a synthetic validation dataset of 20 gear images
(10 perfect / 10 defective) so the inspection pipeline can be tested.
 
This is ONLY needed because we don't have the real factory dataset.
If NUML/DecodeLabs gives you actual images later, just put them in
the 'dataset' folder (named good_1.jpg...good_10.jpg / bad_1.jpg...bad_10.jpg)
and skip this script.
"""
 
import cv2
import numpy as np
import math
import random
import os
 
OUT_DIR = "dataset"
os.makedirs(OUT_DIR, exist_ok=True)
 
IMG_SIZE = 400
CENTER = (IMG_SIZE // 2, IMG_SIZE // 2)
OUTER_R = 150
INNER_R = 120
HOLE_R = 40
NUM_TEETH = 20
 
 
def make_gear_points(broken_tooth_index=None, crack=False):
    """Builds the list of (x, y) points that make up the gear outline."""
    points = []
    angle_step = 2 * math.pi / (NUM_TEETH * 2)
 
    for i in range(NUM_TEETH * 2):
        angle = i * angle_step
        # even index -> tip of tooth (outer radius), odd -> root (inner radius)
        radius = OUTER_R if i % 2 == 0 else INNER_R
 
        # simulate a broken tooth: crush the radius down to the root level
        tooth_num = i // 2
        if broken_tooth_index is not None and tooth_num == broken_tooth_index:
            radius = INNER_R - 25  # tooth snapped off, deeper than normal root
 
        x = CENTER[0] + radius * math.cos(angle)
        y = CENTER[1] + radius * math.sin(angle)
        points.append((x, y))
 
    return np.array(points, dtype=np.int32)
 
 
def draw_gear(defective=False, seed=0):
    random.seed(seed)
    img = np.ones((IMG_SIZE, IMG_SIZE, 3), dtype=np.uint8) * 30  # dark background
 
    broken_index = random.randint(0, NUM_TEETH - 1) if defective else None
    pts = make_gear_points(broken_tooth_index=broken_index)
 
    # metallic grey gear body
    cv2.fillPoly(img, [pts], (170, 170, 175))
    cv2.polylines(img, [pts], isClosed=True, color=(90, 90, 95), thickness=2)
 
    # center shaft hole
    cv2.circle(img, CENTER, HOLE_R, (30, 30, 30), -1)
 
    # small keyway notch (present on every gear, not a defect)
    cv2.rectangle(img, (CENTER[0] - 8, CENTER[1] - HOLE_R - 5),
                  (CENTER[0] + 8, CENTER[1] - HOLE_R + 15), (30, 30, 30), -1)
 
    # a surface crack line for some defective samples (visual variety only)
    if defective and random.random() > 0.5:
        p1 = (CENTER[0] + random.randint(-30, 30), CENTER[1] + random.randint(-30, 30))
        p2 = (p1[0] + random.randint(20, 50), p1[1] + random.randint(20, 50))
        cv2.line(img, p1, p2, (20, 20, 20), 2)
 
    # mild random noise to mimic camera sensor noise
    noise = np.random.randint(0, 12, (IMG_SIZE, IMG_SIZE, 3), dtype=np.uint8)
    img = cv2.add(img, noise)
 
    return img
 
 
def main():
    for i in range(1, 11):
        img = draw_gear(defective=False, seed=i)
        cv2.imwrite(os.path.join(OUT_DIR, f"good_{i}.jpg"), img)
 
    for i in range(1, 11):
        img = draw_gear(defective=True, seed=100 + i)
        cv2.imwrite(os.path.join(OUT_DIR, f"bad_{i}.jpg"), img)
 
    print("Dataset created: 10 good + 10 defective images in 'dataset/' folder")
 
 
if __name__ == "__main__":
    main()